| # CROSS FLOW FAN - AC Motor | LWCA-40800SN-07-00 |
| --- | --- |

Ningbo Longwell Electric Technology CO.,LTD

NO.100, Ditang District ,Yuyao,Zhejiang Province,China 315400 Phone 86 574 6218 9863

Fax 86 574 6218 9863

sales@zjlongwell.com

www.longwellfans.com

| Model No.: | LWCA-40800SN-07-00 |
| --- | --- |
| Version Number: | A0 |
| Make : | Zhang |
| Review : | Guo |
| Approvd : | Ye |
| Date： | 20260728 |

| Version number | Notes Remarks | Reason content | Revised by | Date |
| --- | --- | --- | --- | --- |

— 1 — NINGBO LONGWELL ELECTRIC TECHNOLOGY CO.,LTD

.

LWCA-40800SN-07-00

CROSS FLOW FAN - AC Motor

# Nominal Data

| Project1 | Unit | Rating |
| --- | --- | --- |
| Nominal Voltage : | [VAC] | 230 |
| Frequency : | [Hz] | 50 |
| Speed : | [rpm] | 2200 |
| Power input : | [W] | 22 |
| Air flow (Max) : | [m3/h] | 236 |
|  | [CFM] | 139 |
| Static pressure ( Max) : | [Pa] | 24 |
| Noise level : | dB(A) | 37 |
| Max.ambient Temp : | ℃ | 60 |
| Min.ambient Temp : | ℃ | -25 |
| Insulation Class : |  | Class F |
| Work system : |  | S1 |

NINGBO LONGWELL ELECTRIC TECHNOLOGY CO.,LTD

LWCA-40800SN-07-00

CROSS FLOW FAN - AC Motor

Technical Features

| Material : | Aluminium ( Impeller )； Galvanized sheet（frame） |
| --- | --- |
| Service environment : | Operating temperature：-25~60℃；Operating humidity：5 ~ 85 % RH Storing temperature：-25~60℃；Storing humidity：0 ~ 85 % RH |
| Insulation Class : | Class F（155℃) |
| Insulation Resistance： | Cold resistance between stator core and winding is more than 50 m Ω . (using DC500V Meg-ohmmeter to test). |
| Pressure test: | Motor run 1 seconds, under 1800V/ 2mA ,no breakdown and flash-over. |
| Protected Mode: | Z.P impedance protection. |
| Certification | CE |
| Bearing | Ball bearing |

— 3 — NINGBO LONGWELL ELECTRIC TECHNOLOGY CO.,LTD

LWCA-40800SN-07-00

CROSS FLOW FAN - AC Motor

Product Drawing

NINGBO LONGWELL ELECTRIC TECHNOLOGY CO.,LTD

LWCA-40800SN-07-00

CROSS FLOW FAN - AC Motor

Wiring Diagram

Label

Confirm by sales team when placing sample order or bulk order

— 5 — NINGBO LONGWELL ELECTRIC TECHNOLOGY CO.,LTD

LWCA-40800SN-07-00

CROSS FLOW FAN - AC Motor

Warning:

1.Thefanand motormustbeusedwithintheprescribedscopeoftemperatureand humidity otherwiseit willcauseanunexpecteddamage.

2.Fanisnotsetwrongconnection protection ，Providetheright voltageandlinesequenceaccordingto thenameplateotherwiseitwilldamage theproduct.

3. Wiring depends on the wiring diagram and please connect the earth ground if required. We don't suggesttousetheextensioncord .Any wrongconnectionmay cause thedamageof thefan.

4. Be careful when moving the fan and do not take the blades as a handle because it will cause the distortionandchatter.

5.  Connect  the  thermostat  if  required  otherwise  it  would  damage  the  motor  because  of  the  over

temperature.

6. The  length  of the  mounting  screws  shall  not  exceed the  requirement,  otherwise  the fan will  be

damaged.

7.Donot disassemble the fanarbitrarily.It mayhurt thecapacityofwater tightnessand dynamicbalance orcauseotherserious problems.

8.Theover currentprotector isnecessaryincasethedamagefromtheovercurrent.

9.Installthefanasrequired,anyotherinstalling directionwould affecttheservicelifeoftheproduct.

10.Thefan withinletringshouldbeinstalledfollowtherequirementsorit willaffecttheperformance.

11.Usersshoulduse thestandardcablewheninstall thefan otherwiseitwillaffect the watertightness.

12.Thefanshouldkeepawayfrom thechildrenespeciallythebladeandelectricparts .Andthechildren arenotallowedto operatethe fanalone.

13. Do not connect the white signal cable directly to the red signal cable or the external DC power supply.Otherwise,thefanmaybe damaged.T

14.Pleasedonotperform extractionandinsertionoftheconnectorunderrevolutionirrespectiveof poweronandpoweronandpoweroff , andthe fanshouldnottouchormovewhenrunning.

15. Do not lift the fan wire when taking the fan, and do not impact the bearing during the handling

process.

16 The version  change  may  not  be  notified,  please  ask  our  company for the  latest  version when

necessary.

17.Please be informed that we are not responsible for any damage or accidents caused by violating aboverulesto installandoperatethefan.

— 7 — NINGBO LONGWELL ELECTRIC TECHNOLOGY CO.,LTD

| CROSS FLOW FAN - AC Motor | LWCA-40800SN-07-00 |
| --- | --- |

Approval

DATE :

CUSTOMER APPROVAL

CUSTOMER SIGNATURE:

Product Data Confirm :

— 8 — NINGBO LONGWELL ELECTRIC TECHNOLOGY CO.,LTD